PRODUCT REVIEW

FullSpectrum Feature Summary

Local-Z, gradients, and surface painting

Audience: Product Management   |   Purpose: Feature overview   |   25 August 2026

This feature set reduces configuration friction in mixed-filament workflows, makes layer-height behavior more predictable, and expands control over visible surface results.

| FEATURE | WHAT CHANGED | PRODUCT VALUE |
| --- | --- | --- |
| 01 First-layer height alignment | In SML / Local-Z workflows, the first object layer now follows the configured Initial layer height. If it is subdivided, the combined passes still equal that height. | Predictable first-layer thickness and adhesion, with consistent behavior across standard and mixed-filament prints. |
| 02 Printer-defined Local-Z ceiling | The separate Local-Z upper-height setting has been removed. The upper bound now comes from the printer's maximum layer height for the active filament or extruder. | Fewer settings to manage and a single, printer-aware safety limit. |
| 03 Independent gradient cadence | Gradients can use Local-Z automatically and have their own Gradient Local-Z layer height. Users no longer need to increase the model's process layer height or enable SML just to activate a gradient. | Fine geometry and smooth color transitions can be tuned independently, reducing setup friction. |
| 04 Surface depth for mixed filaments | A surface-only option controls how deeply facet-painted physical, mixed, or gradient filament regions extend into the wall shell, instead of recoloring the full volume. | More predictable surface coloration, less internal material mixing, and better control of appearance versus material use. |
| 05 Rectangle and polygon painting | Rectangle and polygon selection tools complement brush-based painting. Rectangle covers large areas quickly; polygon creates precise, editable boundaries around irregular regions. | Faster painting of large faces and cleaner control on complex shapes. |

Product story: printer limits define safe behavior, gradients work independently, and surface painting gains both depth and selection control.