FULLSPECTRUM TECHNICAL GUIDE

# Local-Z, gradients, surface depth, and painting tools

Current behavior, configuration rules, and practical use

Audience: slicer users, QA, support, and development | Updated: 25 August 2026

In brief: SML/Local-Z now honors the configured first-layer geometry, uses printer-defined maximum layer heights, and lets gradients run on an independent cadence. Painted mixed materials can be constrained to the visible shell and applied efficiently with screen-space rectangle and polygon masks.

Summary of the five changes

| Area | Current behavior | Practical effect |
| --- | --- | --- |
| First layer | Uses the configured Initial layer height. | Bed-adhesion geometry remains predictable. |
| Upper limit | Uses each filament's printer maximum; the Local-Z upper setting is retired. | One authoritative safety limit. |
| Gradients | Automatically use Local-Z with an independent cadence height. | No higher process layer or global SML switch is required. |
| Surface depth | Paint may be shell-limited or use a configured segmentation depth. | Mixed color stays near the visible surface when desired. |
| Painting | Rectangle and polygon masks project from the current view. | Large and irregular regions are faster to paint precisely. |

# Scope and terminology

This guide uses SML/Local-Z to mean the mixed-filament layer-subdivision workflow. A nominal process layer defines the object's sliced geometry. Local-Z may then represent a mixed material with one or more physical-filament passes inside, or across, those nominal intervals.

Important: These controls affect different dimensions. Process and initial layer heights define geometric slicing; Local-Z and gradient cadence heights define how mixed color is distributed in Z; surface-depth controls define how far painted material penetrates inward from the model skin.

# 1  First layer follows Initial layer height

Current rule: The first object layer uses the configured Initial layer height. Local-Z planning consumes that actual first-layer interval; it does not silently replace it with the normal process layer height.

For example, if Initial layer height is 0.24 mm and Layer height is 0.12 mm, the object begins at Z = 0.24 mm. The later nominal layers then follow the 0.12 mm process height.

| Setting | Example | Result |
| --- | --- | --- |
| Initial layer height | 0.24 mm | First object-layer interval totals 0.24 mm |
| Normal layer height | 0.12 mm | Later nominal intervals use 0.12 mm |
| Keep first layer unsplit | Enabled | The first full-domain Local-Z layer remains one 0.24 mm pass |
| Keep first layer unsplit | Disabled | Local-Z may split the first interval, but subpasses still total 0.24 mm |

Keep first layer unsplit is enabled by default for full-domain subdivision. It protects adhesion by preventing thin component passes on the build plate. Painted-only Local-Z regions may still be subdivided according to their scope; even then, the cumulative first-layer Z height remains the configured initial height.

- Set Initial layer height for adhesion and bed-contact requirements, not to tune the gradient.

- Leave Keep first layer unsplit enabled unless a first-layer color split is specifically required and validated.

- Confirm the first layer in Preview by checking its final Z height and any Local-Z subpasses.

# 2  Local-Z upper height is controlled by the printer

Current rule: The user-facing Local-Z upper-height setting has been removed. A physical filament's Printer maximum layer height is the pass-height ceiling for independent Local-Z and gradient cadences.

The old configuration key mixed_filament_height_upper_bound is retired and ignored when legacy projects are loaded. Local-Z minimum sublayer height remains available because it answers a different question: how thin an active component pass is allowed to be.

| Constraint | Source | Behavior |
| --- | --- | --- |
| Minimum pass height | Local-Z minimum sublayer height | Clamps very small active-component contributions; default 0.06 mm, hard minimum 0.01 mm. |
| Maximum pass height | Printer maximum layer height for the selected physical filament | Splits an oversized contribution into legal passes. |
| Maximum fallback | 75% of that filament's nozzle diameter | Used only when the printer profile does not provide a positive maximum. |

An oversized component contribution is divided into equal passes while preserving its total thickness and requested ratio.

Required component thickness: 0.28 mm | Printer maximum: 0.20 mm | Emitted: 0.14 mm + 0.14 mm

- Maintain max_layer_height in the printer profile for every physical filament/extruder used by the mix.

- Expect additional same-filament passes when an extreme ratio produces a contribution above the printer maximum.

- Older projects remain loadable; the obsolete Local-Z upper value no longer controls slicing.

# 3  Gradients are decoupled from process layer height and SML

Current rule: A spatial gradient automatically enters the Local-Z path. It does not require Subdivide all mix layers, and the object's normal process layer height does not need to be increased to fit the gradient passes.

| Control | What it controls | What it no longer controls |
| --- | --- | --- |
| Layer height | Geometric slicing of the model | Whether a gradient can subdivide |
| Gradient Local-Z layer height | Independent A/B color-cadence budget; default 0.20 mm | The model's geometric layer resolution |
| Subdivide all mix layers | Opt-in override for ordinary non-gradient mixed filaments | Gradient activation |
| Full domain for all mixes | Expands full-domain behavior to other eligible painted mixed regions | Object/volume gradients, which receive full-domain handling automatically |

The effective Gradient Local-Z layer height is automatically raised when necessary so two active components can each meet the Local-Z minimum. Gradient passes may cross normal process-layer boundaries. Solid 100/0 and 0/100 endpoints remain single-material rather than being forced into two passes.

Process layer: 0.08 mm | Gradient cadence: 0.20 mm | Requested A/B: 75/25 | Minimum: 0.06 mm

Resulting cadence: 0.14 mm A + 0.06 mm B, carried across process-layer boundaries as needed

A gradient assigned to an entire object or volume automatically receives full-domain treatment. A painted gradient stays inside its painted mask unless Full domain for all mixes expands the eligible area. Physical paint and ordinary non-gradient mixed paint are not pulled into Local-Z merely because a gradient exists elsewhere on the object.

- Choose Layer height for geometry and surface resolution.

- Choose Gradient Local-Z layer height for the color cadence and blend range.

- Use Subdivide all mix layers only when non-gradient mixed rows should also use Local-Z.

# 4  Surface depth with mixed filaments

Purpose: Surface-depth controls determine how far a color-painted physical or mixed filament penetrates into the model. They do not change the mixed recipe, gradient ratio, or Local-Z pass height.

When Surface paint only is enabled, painted side regions are restricted to the configured wall shell. The slicer calculates that shell from the active wall count and wall widths:

shell depth = outer wall width + (wall count - 1) x inner wall width

Example: 0.45 + (3 - 1) x 0.45 = 1.35 mm

| Mode or control | Depth behavior | Use when |
| --- | --- | --- |
| Surface paint only: enabled | Sidewall paint is limited to the calculated wall-shell depth. | Color should remain near the visible surface. |
| Surface paint only: disabled | Maximum width of a segmented region defines penetration; zero disables that limit. | A custom depth or full painted solid region is needed. |
| Extra perimeters in painted zones | Adds 0-8 walls only where paint differs from the parent wall filament; this can increase shell depth. | A thicker colored shell is needed without increasing walls everywhere. |
| Object/volume material assignment | Not restricted by surface-paint depth because it is not a facet-painted mask. | The whole object or volume should use the filament or gradient. |

Painted mixed filaments and painted gradients follow the same surface-depth rule as painted physical filaments. Horizontal painted faces still project through the configured top or bottom shell layers, because those solid layers are part of the visible shell.

Do not confuse depth with bias: Surface depth is inward penetration of a painted mask. Mixed-filament Bias / Selective Expansion contraction is an XY boundary offset that changes how much of a component is exposed at the surface. It does not set paint penetration depth, and grouped wall patterns and Local-Z dithering ignore component bias offsets.

- Beam interlocking bypasses the segmented-region width cut, so strict shell confinement is not guaranteed while it is enabled.

- Very thin features may contain little or no unpainted core because the wall shell occupies most of the cross-section.

- The implementation currently uses the maximum calculated shell width across the object's print regions.

# 5  Rectangle and polygon painting tools

Current behavior: Rectangle mask and Polygon mask are screen-space projection tools in Color Painting. They apply the selected physical or mixed filament to front-facing facets inside the projected shape from the current camera view.

| Tool | Create or edit the mask | Apply, erase, or cancel |
| --- | --- | --- |
| Rectangle mask | Drag from one corner to the opposite corner. | Release to apply. Shift-drag erases. Right-click cancels the active rectangle. |
| Polygon mask | Click to place vertices. Drag an existing vertex to reposition it. At least three vertices are required. | Click the first vertex to close and apply. Hold Shift while closing to erase. Right-click clears/cancels the polygon. |

## Recommended workflow

- Open Color Painting and select the physical or mixed filament that should be applied.

- Set Paint precision. Higher values subdivide boundaries more finely but use more CPU and memory.

- Enable Rectangle mask or Polygon mask. The two mask modes are mutually exclusive.

- Draw the mask from the view that exposes the target faces. Use the clipping plane when interior access is needed.

- Rotate the model and repeat for hidden or rear-facing surfaces, then slice and inspect the material preview.

Both masks respect the active clipping plane and the painter's current filtering options, including overhang-only selection where enabled. Because selection is view-projected, a single mask is intentionally not a through-object paint operation. Each completed action is recorded in the normal undo/redo history.

## Pre-slice checklist

- Verify the intended physical or mixed filament is active before applying the mask.

- Inspect mask edges at the chosen paint-precision level.

- Confirm Surface paint only and Extra perimeters in painted zones match the desired colored-shell depth.

- Inspect the first layer, Local-Z subpasses, toolchanges, and wipe-tower behavior in Preview before printing.

# Key settings reference

| UI label | Configuration key | Role |
| --- | --- | --- |
| Initial layer height | initial_layer_print_height | Total first object-layer height |
| Layer height | layer_height | Normal geometric slicing height |
| Keep first layer unsplit | dithering_local_z_preserve_first_layer | Protects the first full-domain Local-Z layer |
| Local-Z minimum sublayer height | mixed_filament_height_lower_bound | Minimum active-component pass height |
| Printer maximum layer height | max_layer_height | Maximum pass height per physical filament/extruder |
| Gradient Local-Z layer height | dithering_local_z_gradient_layer_height | Independent gradient cadence budget |
| Subdivide all mix layers | dithering_local_z_mode | Global override for ordinary non-gradient mixed rows |
| Full domain for all mixes | dithering_local_z_whole_objects | Expands full-domain behavior to other eligible mixed regions |
| Surface paint only | fs_surface_paint_only | Restricts facet paint to the wall shell |
| Maximum width of a segmented region | mmu_segmented_region_max_width | Paint penetration when surface-only mode is off |
| Extra perimeters in painted zones | fs_painted_zone_extra_perimeters | Adds painted-zone walls without changing the rest of the object |

## Implementation references

src/libslic3r/PrintConfig.cpp - option labels, defaults, tooltips, and legacy-key retirement

src/libslic3r/PrintObjectSlice.cpp - Local-Z interval planning, independent gradient cadence, and printer maximum handling

src/libslic3r/MultiMaterialSegmentation.cpp - painted-region depth and Surface paint only shell calculation

src/slic3r/GUI/Gizmos/GLGizmoPainterBase.cpp - rectangle and polygon projection behavior

src/slic3r/GUI/Gizmos/GLGizmoMmuSegmentation.cpp - Color Painting controls and user gestures

tests/libslic3r/test_mixed_filament.cpp and tests/fff_print/test_print.cpp - regression coverage for Local-Z and gradients

Validation note: Local-Z, gradients, and mixed-material painting remain model- and material-sensitive. Treat the sliced preview and a controlled test print as part of acceptance whenever a new combination of ratios, layer heights, or paint depth is used.